What are the type of memory failure ?

Types of Memory Failures

For memories like:

- NAND Flash

- EEPROM

- SRAM

- DRAM

- FPGA BRAM

memory failures are usually classified by:

- behavior,

- physical cause,

- persistence,

- radiation mechanism.

1. Bit Flip (Single-Bit Upset)

What Happens

One stored bit changes:

1 → 0

or

0 → 1

Cause

Usually:

- SEU,

- charge disturbance,

- noise,

- weak retention.

Example

Expected:

5555

Read:

5551

Characteristics

| Property | Behavior |
| --- | --- |
| Localized | Yes |
| Temporary | Often |
| Recoverable | Usually |
| Radiation relation | SEU |

2. Multi-Bit Upset (MBU)

What Happens

Multiple nearby bits flip simultaneously.

Example:

10101010

→

10011010

Cause

Single particle deposits charge across multiple cells.

More common in:

- high-density NAND,

- advanced nodes.

Characteristics

| Property | Behavior |
| --- | --- |
| Adjacent errors | Yes |
| Clustered | Yes |
| Radiation relation | Heavy-ion/proton SEE |

3. Stuck-at Fault

What Happens

Bit permanently stuck:

always 0

or

always 1

Cause

- oxide damage,

- transistor failure,

- severe TID,

- physical damage.

Example

Write:

1

Read:

0

every time.

4. Retention Failure

What Happens

Data initially correct,
but changes over time.

Cause

Charge leakage from:

- floating gate,

- damaged oxide,

- displacement defects.

Example

write → wait → corrupted readback

Strongly Associated With

- TID,

- DD,

- NAND wear.

5. Read Disturb Failure

What Happens

Repeated reading changes nearby cell charge.

Cause

Electric field coupling during repeated reads.

Radiation can worsen sensitivity.

Common In

- NAND Flash.

6. Program Disturb / Write Disturb

What Happens

Programming one cell alters neighboring cells.

Cause

Capacitive coupling,
high electric fields.

Signature

Unexpected nearby corruption after write operations.

7. Address Decoder Failure

What Happens

Wrong memory location accessed.

Example

Write:

Address 0001 = AAAA

Read:

Address 0001 = value from another location

Cause

- SEFI,

- logic upset,

- decoder damage.

8. Functional Interrupt (SEFI)

What Happens

Memory device stops functioning properly.

Symptoms

- no response,

- invalid outputs,

- interface lockup,

- requires reset.

Cause

Upset in:

- internal controller,

- state machine,

- command logic.

9. Timing Failure

What Happens

Read/write timing becomes invalid.

Cause

- TID parameter drift,

- degraded transistor speed.

Symptoms

- intermittent failures,

- communication instability.

10. Block Failure

What Happens

Entire NAND block/page becomes unusable.

Cause

- severe oxide degradation,

- accumulated TID,

- endurance wear,

- DD.

Symptoms

ECC cannot recover data.

11. Leakage Failure

What Happens

Memory current consumption increases.

Cause

- oxide trapped charge,

- junction leakage,

- TID.

Signature

Abnormal standby/supply current.

12. Catastrophic Failure

What Happens

Memory permanently dead.

Cause

- SEL destruction,

- SEGR,

- SEB,

- extreme TID.

Classification by Persistence

| Failure Type | Temporary | Permanent |
| --- | --- | --- |
| SEU | Yes | Usually no |
| MBU | Yes | Usually no |
| SEFI | Often | Sometimes |
| Retention failure | Sometimes | Can become permanent |
| Stuck-at fault | No | Yes |
| Block failure | No | Yes |
| SEL damage | No | Yes |

Classification by Radiation Mechanism

| Radiation Effect | Typical Memory Failure |
| --- | --- |
| SEU | Bit flip |
| SET | Transient wrong output |
| SEFI | Controller hang |
| TID | Retention/timing/leakage failure |
| DD | Retention degradation |
| SEL | Destructive overcurrent |

Most Important Failures for Your System

EEPROM

Main concerns:

- SEU

- retention failure

- SEFI

- stuck bits

NAND Flash

Main concerns:

- MBU

- retention degradation

- block failure

- ECC growth

- SEFI

FPGA SRAM

Main concerns:

- SEU

- configuration corruption

- logic upset

- transient faults