1. NAND Flash

Main radiation sensitivities:

| Effect | Physical Defect Mechanism |
| --- | --- |
| TID | Charge trapping in tunnel oxide/inter-cell dielectric |
| SEU | Charge loss/gain in floating gate |
| DD | Lattice damage causing leakage/recombination |
| SEFI | Internal controller/state-machine disruption |
| SEL | Rare but possible in peripheral CMOS circuits |

NAND Flash — Required Tests

| Test | Detects | Methodology |
| --- | --- | --- |
| Pattern Retention Test | TID, DD | Write known pattern → wait → read back |
| Read/Write Verify Test | SEU, TID | Continuous write/read/compare |
| ECC Statistics Monitoring | Early degradation | Monitor correction count increase |
| Bad Block Growth Test | TID, DD | Track new unusable blocks over dose |
| Endurance Cycling | Oxide degradation | Repeated P/E cycles during irradiation |
| Current Monitoring | SEL/TID leakage | Measure supply current continuously |
| SEFI Recovery Test | SEFI | Check if reset required after upset |
| Multi-Block Pattern Test | Large-area corruption | Write different patterns across blocks |

Important NAND Methodologies

A. Checkerboard Pattern Test

Write:

5555
AAAA

across entire NAND.

Detects:

- SEU,

- MBU,

- retention loss,

- weak cells.

Signature:

- readback mismatch.

B. Retention Test

Method:

- Write pattern

- Irradiate

- Wait:

  - minutes,

  - hours,

  - days

- Read back

Detects:

- oxide leakage,

- trapped charge effects,

- TID-induced retention degradation.

C. ECC Growth Monitoring

Modern NAND internally corrects errors.

Radiation damage first appears as:

- increased ECC corrections,
before catastrophic failure.

Important metric:

Corrected bit errors per MB

D. Program/Erase Stress Test

Method:

Repeated:

erase → write → verify

during irradiation.

Detects:

- tunnel oxide wear acceleration,

- TID-induced write degradation.

E. Address Pattern Test

Store:

address = data

Detects:

- decoder corruption,

- addressing faults,

- SEFI behavior.

NAND Failure Signatures

| Observation | Likely Radiation Effect |
| --- | --- |
| Single bit error | SEU |
| Adjacent bit corruption | MBU |
| Increasing ECC corrections | Early TID/DD |
| Entire block failure | Severe TID/DD |
| Controller freeze | SEFI |
| High current | SEL |
| Retention loss over time | Oxide trapping/TID |

2. EEPROM

EEPROM is usually:

- floating-gate,

- smaller density,

- more robust than NAND,
but still radiation sensitive.

EEPROM Radiation Defects

| Effect | Physical Mechanism |
| --- | --- |
| TID | Oxide trapped charge shifts threshold |
| SEU | Floating-gate charge disturbance |
| SEFI | Peripheral/control logic upset |
| DD | Charge retention degradation |
| SEL | CMOS peripheral latch-up |

EEPROM Required Tests

| Test | Detects | Method |
| --- | --- | --- |
| Static Pattern Readback | SEU | Write pattern → read compare |
| Retention Test | TID/DD | Long-delay readback |
| Write/Erase Verification | TID degradation | Verify programming success |
| Functional Access Test | SEFI | Continuous read/write operation |
| Current Monitoring | SEL | Measure supply current |
| Address Sweep Test | Decoder upset | Sequential address verification |

EEPROM Methodologies

A. Static Pattern Test

Usually:

0000
FFFF
5555
AAAA

Why:

EEPROM often experiences:

- isolated bit upsets,

- retention weakening.

Signature:

- wrong readback value.

B. Long Retention Test

EEPROM may pass immediately after write,
but fail later due to:

- radiation-induced leakage.

Method:

write → irradiate → wait → verify

Very important for TID studies.

C. Read-Disturb Monitoring

Repeated reading itself may worsen weak cells.

Radiation can accelerate this.

Method:

- repeatedly read same locations,

- observe bit degradation.

D. Functional Cycling Test

Continuous:

write → read → erase → verify

during irradiation.

Detects:

- peripheral timing degradation,

- SEFI,

- write failure.

Recommended Best-Practice Methodology

For both NAND and EEPROM:

1. Baseline Characterization

Before irradiation measure:

- current,

- BER,

- retention,

- write time,

- ECC count.

2. Dynamic Irradiation Test

Operate device live during radiation:

- continuous pattern checking,

- current monitoring,

- telemetry logging.

3. Post-Irradiation Retention Test

Very important.

Some defects appear:

- hours later,

- not immediately.

4. Event Logging

Log:

- address,

- expected data,

- observed data,

- timestamp,

- radiation dose/LET,

- current.

Most Important Thing

For memories:

The methodology is not just “did it fail?”

It is:

- how many bits failed,

- where,

- transient or permanent,

- recoverable or not,

- retention or immediate,

- random or clustered.

That distinction identifies:

- SEU,

- MBU,

- TID,

- DD,

- SEFI,
separately.

ADC Radiation-Induced Failures

For an ADC, radiation usually affects:

- analog front-end,

- comparators,

- reference circuitry,

- sampling capacitors,

- digital control/output logic.

The failures are different from memory failures because ADCs are:

mixed-signal devices.

Main ADC Failure Types

| Failure Type | What Happens | Radiation Cause | Observable Signature |
| --- | --- | --- | --- |
| Conversion Error | Wrong digital output value | SEU, TID, SET | ADC code differs from expected value |
| Transient Spike | Temporary incorrect sample | SET | Sudden abnormal output sample |
| Offset Drift | Output shifted from true value | TID | Constant measurement shift |
| Gain Drift | Scaling changes | TID/DD | ADC slope changes |
| Noise Increase | Random fluctuation increases | TID/DD | Larger output variance |
| Missing Codes | Certain output codes absent | Comparator/reference degradation | Nonlinear transfer curve |
| Nonlinearity | ADC transfer deviates | TID | INL/DNL degradation |
| SEFI | ADC freezes/stops converting | SEE | No output update until reset |
| Timing Failure | Sampling instability | TID | Irregular conversion timing |
| Leakage Increase | Higher current consumption | TID | Increased supply current |
| Catastrophic Failure | ADC permanently damaged | SEL/SEB | No functionality |

Important Radiation Effects in ADCs

| Radiation Effect | Typical ADC Behavior |
| --- | --- |
| SEU | Wrong digital output word |
| SET | Single transient conversion spike |
| SEFI | ADC hangs/freezes |
| TID | Drift/noise/accuracy degradation |
| DD | Analog performance degradation |
| SEL | Sustained overcurrent |
| SEGR/SEB | Rare unless power stages involved |

ADC Test Methodologies

1. Fixed Analog Input Test

Method

Apply stable reference voltage:

0.5V

1.0V

2.0V

continuously sample ADC.

Detects

| Failure | Signature |
| --- | --- |
| SEU | Wrong code |
| SET | Sudden spike |
| TID | Gradual drift |
| Noise increase | Larger variation |

Example

Expected:

2048

2048

2048

2048

Observed:

2048

2047

3150

2048

The 3150 sample may indicate:

SET or SEU.

2. Ramp Test

Method

Apply slowly increasing analog voltage.

Example:

0V → Full scale

Detects

- missing codes,

- INL/DNL degradation,

- nonlinear behavior.

Radiation Signatures

| Observation | Possible Cause |
| --- | --- |
| Missing output codes | Comparator degradation |
| Uneven transitions | TID |
| Hysteresis | Analog front-end damage |

3. Sine-Wave Test

Method

Apply sinusoidal input.

Analyze:

- FFT,

- SNR,

- THD,

- ENOB.

Detects

| Parameter | Radiation Impact |
| --- | --- |
| SNR | Noise increase |
| ENOB | Accuracy reduction |
| THD | Analog distortion |
| SFDR | Spurious effects |

4. Continuous Sampling Test

Method

ADC continuously converts known signal.

FPGA/software checks:

measured value == expected range

Detects

- SEFI,

- transient spikes,

- frozen outputs.

5. Cross-Comparison Method

Useful in your setup.

Example:

DAC → OpAmp → ADC

Known DAC value should produce:
known ADC code.

Detects

| Device Failure | Signature |
| --- | --- |
| DAC upset | Wrong analog level |
| ADC upset | Wrong digital code |
| OpAmp drift | Shifted response |
| SET | Temporary mismatch |

6. Current Monitoring

Detects

- SEL,

- TID leakage.

Signature

| Observation | Meaning |
| --- | --- |
| Sustained overcurrent | SEL |
| Gradual current rise | TID leakage |

Most Important ADC Radiation Failure

Usually:

SET (Single Event Transient)

Because ADCs are analog-sensitive.

A particle strike can create:

- one abnormal sample,

- transient spike,

- corrupted conversion.

Typical SET Signature

Normal:

2048

2049

2048

2048

SET:

2048

4095

2048

2048

One large isolated spike.

Best Detection Strategy for Your System

For your architecture:

| Technique | Importance |
| --- | --- |
| Fixed known voltage | Essential |
| Continuous sampling | Essential |
| Threshold comparison | Essential |
| Timestamp logging | Very important |
| Spike detection | Important |
| Current monitoring | Important |
| Long-term drift monitoring | Important for TID |

Good Practical Criterion

Example:

|Measured - Expected| > Threshold

Then:

- flag radiation event.

You can classify:

| Behavior | Likely Effect |
| --- | --- |
| One abnormal sample | SET |
| Persistent offset | TID |
| Frozen output | SEFI |
| Overcurrent | SEL |
| Increased noise | DD/TID |

DAC Radiation-Induced Failures

A DAC is sensitive because it contains:

- digital registers,

- reference circuitry,

- resistor/capacitor networks,

- analog output stages.

Radiation can affect:

- digital data,

- analog accuracy,

- output stability,

- internal control logic.

Main DAC Failure Types

| Failure Type | What Happens | Radiation Cause | Observable Signature |
| --- | --- | --- | --- |
| Wrong Output Code | DAC outputs incorrect voltage | SEU | Output voltage differs from expected |
| Transient Output Spike | Short abnormal analog pulse | SET | Sudden voltage glitch |
| Offset Drift | Entire output shifted | TID | Constant output error |
| Gain Drift | Output scaling changes | TID/DD | Slope deviation |
| Nonlinearity | Transfer curve distortion | TID | INL/DNL degradation |
| Noise Increase | Output becomes noisy | TID/DD | Voltage fluctuation |
| Stuck Output | Output frozen at one level | SEFI | No response to input changes |
| Slow Settling | Output response delayed | TID | Increased settling time |
| Leakage Increase | More supply current | TID | Higher current consumption |
| Catastrophic Failure | DAC permanently damaged | SEL/SEB | No valid output |

Important Radiation Effects in DACs

| Radiation Effect | Typical DAC Behavior |
| --- | --- |
| SEU | Wrong digital-to-analog conversion |
| SET | Transient output glitch |
| SEFI | DAC stops updating |
| TID | Offset/gain drift |
| DD | Analog degradation/noise |
| SEL | Sustained overcurrent |
| SEGR/SEB | Rare unless power stages involved |

DAC Test Methodologies

1. Fixed Digital Pattern Test

Method

Apply known digital values:

0000

5555

AAAA

FFFF

Measure output voltage continuously.

Detects

| Failure | Signature |
| --- | --- |
| SEU | Wrong output voltage |
| SET | Sudden transient glitch |
| TID | Gradual drift |
| SEFI | Output stops changing |

Example

Expected:

5555 → 0.2117V

Observed:

5555 → 0.35V

Possible:

- DAC upset,

- reference drift,

- analog degradation.

2. Step Response Test

Method

Switch between:

low → high → low

Monitor output waveform.

Detects

| Failure | Signature |
| --- | --- |
| SET | Glitch pulse |
| TID | Slower settling |
| Analog degradation | Overshoot/distortion |

3. Ramp Test

Method

Increment DAC code gradually:

0000 → FFFF

Measure transfer curve.

Detects

| Parameter | Radiation Signature |
| --- | --- |
| INL | Transfer distortion |
| DNL | Uneven code steps |
| Missing levels | Internal analog degradation |

4. Continuous Loopback Test

Very useful in your architecture.

Example:

DAC → OpAmp → ADC

Compare:

Expected ADC value

vs

Measured ADC value

Detects

| Device Problem | Signature |
| --- | --- |
| DAC upset | Wrong analog level |
| SET | One abnormal sample |
| SEFI | Frozen output |
| TID | Gradual mismatch increase |

5. Waveform Generation Test

Method

Generate:

- sine wave,

- square wave,

- sawtooth.

Analyze output quality.

Detects

| Parameter | Radiation Impact |
| --- | --- |
| THD | Analog degradation |
| Noise | Increased fluctuation |
| Frequency stability | Timing degradation |

6. Current Monitoring

Detects

- SEL,

- TID leakage.

Signature

| Observation | Meaning |
| --- | --- |
| Sustained overcurrent | SEL |
| Gradual current increase | TID leakage |

Most Important DAC Radiation Failure

Usually:

SET (output glitch)

and

TID drift.

Because DACs are:

- analog-sensitive,

- reference-sensitive.

Typical SET Signature

Normal:

1.000V

1.001V

1.000V

SET:

1.000V

2.8V

1.000V

One transient spike.

DAC vs ADC Radiation Behavior

| DAC | ADC |
| --- | --- |
| Wrong voltage generation | Wrong voltage conversion |
| Output glitch | Conversion glitch |
| Offset/gain drift | Offset/gain drift |
| Frozen output | Frozen conversion |
| Analog output distortion | Digital output corruption |

Best Detection Strategy for Your System

For your setup:

| Technique | Importance |
| --- | --- |
| Known digital patterns | Essential |
| ADC feedback verification | Essential |
| Threshold comparison | Essential |
| Spike detection | Important |
| Current monitoring | Important |
| Drift logging over dose | Important |

Practical Event Interpretation

| Observation | Likely Effect |
| --- | --- |
| One wrong sample | SET |
| Persistent wrong voltage | SEU/TID |
| Gradual drift | TID |
| Frozen output | SEFI |
| Sustained overcurrent | SEL |
| Increased noise | DD/TID |

MOSFET Radiation-Induced Failures

MOSFETs are highly radiation-sensitive because they rely on:

- thin gate oxide,

- electric fields,

- semiconductor junctions.

Power MOSFETs are especially vulnerable to:

- destructive single-event effects.

Main MOSFET Failure Types

| Failure Type | What Happens | Radiation Cause | Observable Signature |
| --- | --- | --- | --- |
| Threshold Voltage Shift | Gate threshold changes | TID | MOSFET turns ON/OFF at wrong voltage |
| Leakage Current Increase | OFF-state current rises | TID, SEGR | Higher drain/gate leakage |
| RDS(on) Increase | ON resistance increases | TID/DD | More voltage drop/heating |
| Switching Degradation | Slower switching | TID/DD | Delayed rise/fall time |
| Transient Pulse | Temporary switching glitch | SET | Unexpected pulse/noise |
| SEL-like Overcurrent | Parasitic conduction | SEE | Sustained abnormal current |
| SEGR (Single Event Gate Rupture) | Gate oxide permanently damaged | Heavy-ion SEE | Permanent gate leakage |
| SEB (Single Event Burnout) | Avalanche thermal destruction | Heavy-ion SEE | Catastrophic short circuit |
| Thermal Runaway | Self-heating increases current | TID/SEB | Increasing temperature/current |
| Catastrophic Failure | Permanent destruction | SEB/SEGR | Device no longer functional |

Important Radiation Effects in MOSFETs

| Radiation Effect | Typical MOSFET Behavior |
| --- | --- |
| TID | Threshold shift, leakage increase |
| DD | Mobility degradation, switching loss |
| SET | Transient switching glitch |
| SEL | Sustained overcurrent (less common than CMOS ICs) |
| SEGR | Gate oxide rupture |
| SEB | Catastrophic burnout |

Most Dangerous MOSFET Effects

1. SEGR — Single Event Gate Rupture

What Happens

Heavy ion strikes gate oxide while:

- high electric field exists.

Gate oxide breaks permanently.

Result

Gate leakage becomes very high

MOSFET may:

- partially work,

- fail permanently,

- become unstable.

Signature

| Observation | Meaning |
| --- | --- |
| Permanent gate leakage | SEGR |
| Gate no longer controls channel properly | Oxide rupture |

2. SEB — Single Event Burnout

What Happens

Particle strike causes:

- localized avalanche conduction,

- thermal runaway.

MOSFET physically burns internally.

Result

Usually catastrophic:

Drain-source short

Signature

| Observation | Meaning |
| --- | --- |
| Sudden huge current | SEB |
| Permanent short circuit | Burnout |
| Thermal destruction | Catastrophic SEE |

MOSFET Test Methodologies

1. Gate Threshold Test

Method

Sweep gate voltage:

VGS

Measure drain current.

Detects

| Failure | Signature |
| --- | --- |
| TID | Threshold shift |
| Oxide damage | Abnormal transfer curve |

Example

Before radiation:

VTH = 2.5V

After:

VTH = 1.8V

2. Leakage Current Test

Method

MOSFET OFF-state:

VGS = 0

Measure:

IDSS

IGSS

Detects

| Failure | Signature |
| --- | --- |
| TID | Drain leakage increase |
| SEGR | Gate leakage increase |

Very Important

Gate leakage rise:

IGSS ↑

is one of the strongest indicators of:

SEGR.

3. Switching Test

Method

Apply PWM/square-wave gate drive.

Observe:

- drain waveform,

- switching speed.

Detects

| Failure | Signature |
| --- | --- |
| SET | Transient glitch |
| DD/TID | Slower switching |
| Gate damage | Distorted switching |

4. Load Switching Test

Method

Operate MOSFET with real load.

Monitor:

- voltage,

- current,

- temperature.

Detects

| Failure | Signature |
| --- | --- |
| SEL-like conduction | Sustained overcurrent |
| SEB | Sudden catastrophic current |
| Thermal runaway | Temperature rise |

5. SOA Stress Test

SOA:

Safe Operating Area

Method

Bias MOSFET near:

- high voltage,

- high current.

during irradiation.

Detects

- SEB sensitivity,

- destructive SEE threshold.

6. Current Monitoring

Critical for MOSFET radiation testing.

Signature Interpretation

| Observation | Likely Effect |
| --- | --- |
| Small transient spike | SET |
| Sustained overcurrent | SEL/parasitic conduction |
| Massive sudden current | SEB |
| Gradual leakage increase | TID |
| Permanent gate leakage | SEGR |

Best Practical Detection Methods

For your architecture:

| Test | Importance |
| --- | --- |
| Current monitoring | Essential |
| Gate leakage measurement | Essential |
| Threshold voltage tracking | Very important |
| Load switching waveform | Important |
| Thermal monitoring | Important |
| Automatic shutdown protection | Important |

Most Important Measurements

| Parameter | Why Important |
| --- | --- |
| VTH | TID indicator |
| IGSS | SEGR indicator |
| IDSS | Leakage/TID indicator |
| Drain current | SEL/SEB indicator |
| Temperature | Thermal runaway indicator |

Practical Interpretation

| Observation | Likely Radiation Effect |
| --- | --- |
| Threshold drift | TID |
| Increased OFF leakage | TID/DD |
| One switching glitch | SET |
| Sustained overcurrent | SEL-like event |
| Permanent gate leakage | SEGR |
| Permanent short circuit | SEB |
| Device explosion/burnout | Catastrophic SEB |

FPGA Radiation-Induced Failures

For FPGAs, radiation affects:

- configuration memory,

- flip-flops/registers,

- BRAM,

- routing fabric,

- clock networks,

- transceivers,

- control/state machines.

Radiation behavior depends strongly on FPGA type:

| FPGA Type | Radiation Sensitivity |
| --- | --- |
| SRAM FPGA | Very high SEU sensitivity |
| Flash FPGA | Lower configuration upset sensitivity |
| Antifuse FPGA | Very radiation tolerant |

Your:

- SmartFusion2 FPGA

- PolarFire FPGA

are flash-based FPGAs, so:

- configuration memory is more robust,

- but user SRAM, BRAM, registers, logic, PLLs still experience SEE/TID effects.

Main FPGA Failure Types

| Failure Type | What Happens | Radiation Cause | Observable Signature |
| --- | --- | --- | --- |
| Configuration Upset | FPGA configuration bit changes | SEU | Wrong logic behavior |
| Register/Flip-Flop Upset | Stored logic state flips | SEU | Counter/state error |
| BRAM Corruption | Internal memory data changes | SEU | Memory mismatch |
| SET (Transient Glitch) | Temporary pulse in logic/routing | SET | Glitch/wrong pulse |
| SEFI | FPGA subsystem hangs | SEE | Device freeze/reset needed |
| Clock Network Failure | PLL/clock disturbed | SEE/TID | Timing instability |
| Timing Degradation | Logic delay changes | TID | Setup/hold violations |
| Leakage Increase | Current rises | TID | Higher supply current |
| SEL | Parasitic latch-up | SEE | Sustained overcurrent |
| Functional Corruption | State machine malfunction | SEU/SET | Incorrect outputs |
| Transceiver Errors | Serial link corruption | SEE | CRC/packet errors |
| Permanent Damage | Severe destructive SEE | SEGR/SEB (rare) | Nonfunctional FPGA |

Most Important FPGA Radiation Effects

| Effect | Typical FPGA Impact |
| --- | --- |
| SEU | Bit flip in logic/register/BRAM |
| SET | Temporary logic glitch |
| SEFI | FPGA freeze/hang |
| TID | Timing/current degradation |
| SEL | Sustained overcurrent |
| DD | Usually less dominant than TID/SEE |

FPGA Test Methodologies

1. Counter Test

Method

Run continuous counters:

0 → 1 → 2 → 3 ...

Monitor correctness.

Detects

| Failure | Signature |
| --- | --- |
| SEU | Wrong count |
| SET | Counter jump |
| SEFI | Counter freeze |

Example

Expected:

100

101

102

103

Observed:

100

101

155

103

Possible:

SEU or SET.

2. BRAM Pattern Test

Method

Write:

5555

AAAA

FFFF

0000

into BRAM.

Continuously read/verify.

Detects

| Failure | Signature |
| --- | --- |
| SEU | Bit mismatch |
| MBU | Adjacent corruption |
| SEFI | Read failure |

3. Logic Loopback Test

Method

Route FPGA outputs back into inputs.

Verify:

input == output

continuously.

Detects

| Failure | Signature |
| --- | --- |
| Routing upset | Wrong logic |
| SET | Transient mismatch |
| SEFI | Stalled loop |

4. PRBS Communication Test

Very important for PolarFire SERDES/transceivers.

Method

Transmit:

PRBS7

PRBS15

PRBS31

Check:

- BER,

- CRC,

- packet integrity.

Detects

| Failure | Signature |
| --- | --- |
| SET | Transient packet error |
| SEU | Persistent corruption |
| Clock upset | BER increase |

5. Watchdog/Heartbeat Test

Method

FPGA periodically toggles:

heartbeat signal

external monitor verifies activity.

Detects

| Failure | Signature |
| --- | --- |
| SEFI | Heartbeat stops |
| Clock failure | Irregular timing |

6. Current Monitoring

Critical for SEL detection.

Signature

| Observation | Meaning |
| --- | --- |
| Short spike | SET/transient |
| Sustained overcurrent | SEL |
| Gradual current increase | TID leakage |

7. CRC / Configuration Verification

Important for FPGA configuration integrity.

Method

Continuously check:

configuration CRC

Detects

| Failure | Signature |
| --- | --- |
| Configuration upset | CRC mismatch |
| Corrupted fabric | Logic malfunction |

8. Timing Margin Test

Method

Run FPGA near timing limits.

Monitor:

- setup violations,

- clock stability.

Detects

| Failure | Signature |
| --- | --- |
| TID | Timing degradation |
| PLL upset | Clock instability |

SmartFusion2 vs PolarFire Radiation Behavior

| Feature | SmartFusion2 | PolarFire |
| --- | --- | --- |
| Configuration storage | Flash | Flash |
| SEU resistance | Good | Better |
| Power consumption | Higher | Lower |
| SERDES sensitivity | Moderate | Important |
| TID robustness | Good | Good |
| User SRAM sensitivity | Present | Present |

Most Important Failures for Your Setup

SmartFusion2

Main risks:

- SEFI

- BRAM SEU

- logic upset

- clock disturbance

- SEL monitoring

PolarFire

Main risks:

- transceiver upset

- SERDES BER increase

- BRAM SEU

- transient logic upset

- SEFI

Best Detection Strategy for Your Architecture

You already use many correct methods.

Strong Methods in Your Design

| Method | Detects |
| --- | --- |
| Checkerboard patterns | BRAM SEU |
| Loopback verification | Logic/routing upset |
| Cross-monitoring FPGA | SEFI |
| Current limiter + FLT | SEL |
| Telemetry/status bits | Functional corruption |
| DAC/ADC verification | Interface/system upset |

Biggest Missing Item

Probably:

high-speed transient capture

Because many FPGA SETs last:

- ns to µs.

Slow polling may miss them.

Practical Radiation Event Interpretation

| Observation | Likely FPGA Radiation Effect |
| --- | --- |
| Wrong memory value | SEU |
| One transient logic error | SET |
| Frozen FPGA | SEFI |
| Sustained overcurrent | SEL |
| CRC mismatch | Configuration upset |
| Timing instability | TID |
| BER increase | Transceiver upset |
| Gradual current rise | TID leakage |

Crystal Oscillator (XTAL Oscillator) Radiation-Induced Failures

A crystal oscillator contains:

- quartz crystal resonator,

- oscillator amplifier,

- feedback network,

- bias circuitry,

- sometimes PLL/clock conditioning circuitry.

Radiation can affect:

- the crystal itself,

- the oscillator electronics.

Usually the oscillator IC electronics are more sensitive than the quartz crystal.

Main XTAL Oscillator Failure Types

| Failure Type | What Happens | Radiation Cause | Observable Signature |
| --- | --- | --- | --- |
| Frequency Drift | Output frequency shifts | TID, DD | Measured frequency changes |
| Increased Jitter | Clock timing becomes unstable | TID, SET | Timing variation/noisy clock |
| Missing Pulses | Clock pulses disappear temporarily | SET | Pulse dropout |
| Startup Failure | Oscillator fails to start | TID/DD | No oscillation |
| Amplitude Reduction | Output swing decreases | TID | Lower clock amplitude |
| SEFI-like Lockup | Oscillator stops oscillating | SEE | Clock halts until reset |
| PLL Instability | PLL loses lock | SET/TID | Unstable derived clocks |
| Phase Noise Increase | Frequency purity degrades | DD/TID | Noisy spectrum |
| Leakage Increase | Higher current consumption | TID | Increased supply current |
| Permanent Failure | Oscillator dead | Severe TID/SEE | No output |

Important Radiation Effects in Oscillators

| Radiation Effect | Typical Oscillator Behavior |
| --- | --- |
| TID | Frequency drift, startup degradation |
| DD | Crystal parameter degradation |
| SET | Temporary clock glitch |
| SEFI | Oscillation halt |
| SEL | Possible in oscillator IC |
| SEU | Less relevant unless digital synthesizer used |

Most Important Oscillator Radiation Problems

Usually:

frequency instability

and

transient clock disturbance.

Because clocks affect:

- FPGA timing,

- ADC sampling,

- communication synchronization,

- system stability.

XTAL Oscillator Test Methodologies

1. Frequency Monitoring Test

Method

Continuously measure:

output frequency

using:

- frequency counter,

- FPGA counter,

- reference clock comparison.

Detects

| Failure | Signature |
| --- | --- |
| TID | Gradual frequency drift |
| DD | Long-term instability |
| SET | Sudden short deviation |

Example

Expected:

10.000000 MHz

Observed:

9.998500 MHz

Indicates:

- frequency drift,

- possible TID/DD degradation.

2. Pulse Monitoring Test

Method

Count clock edges continuously.

Verify:

expected pulse count

Detects

| Failure | Signature |
| --- | --- |
| SET | Missing/extra pulse |
| SEFI | Clock stops |

Example

Expected:

100000 pulses

Observed:

99995 pulses

Possible transient upset.

3. Jitter Measurement

Method

Measure:

- cycle-to-cycle variation,

- phase noise,

- timing uncertainty.

Detects

| Failure | Signature |
| --- | --- |
| TID | Increased jitter |
| DD | Noisy timing |
| SET | Isolated timing spike |

4. Startup Reliability Test

Method

Repeated:

power ON → verify oscillation startup

during/after irradiation.

Detects

| Failure | Signature |
| --- | --- |
| TID | Startup failure |
| Bias degradation | Delayed oscillation |

5. PLL Lock Monitoring

If oscillator feeds PLL.

Method

Monitor:

PLL lock signal

Detects

| Failure | Signature |
| --- | --- |
| SET | Temporary unlock |
| TID | Increasing instability |

6. Current Monitoring

Detects

- SEL,

- TID leakage.

Signature

| Observation | Meaning |
| --- | --- |
| Sustained overcurrent | SEL |
| Gradual current increase | TID leakage |

Most Important Practical Detection Method

For your architecture:

Pulse-count comparison

is probably the best practical method.

Example FPGA-Based Method

Use:

- reference stable clock,

- counter driven by XTAL oscillator.

Every interval:

compare expected count vs actual count

Detects

| Failure | Signature |
| --- | --- |
| Frequency drift | Count deviation |
| Missing pulses | Count loss |
| Oscillator stop | Zero count |
| Jitter burst | Irregular timing |

Typical Radiation Event Signatures

| Observation | Likely Effect |
| --- | --- |
| One missing pulse | SET |
| Gradual frequency drift | TID/DD |
| Oscillator stops | SEFI |
| PLL unlock | Clock transient |
| Increased jitter | Analog degradation |
| High current | SEL |
| Permanent no-output | Severe damage |

Crystal vs Oscillator Electronics

Important distinction:

| Part | Radiation Sensitivity |
| --- | --- |
| Quartz crystal | Relatively robust |
| Oscillator IC electronics | More sensitive |

Usually failures come from:

- CMOS oscillator amplifier,

- bias circuitry,

- PLL circuitry,
not the quartz itself.

Best Detection Strategy for Your System

| Technique | Importance |
| --- | --- |
| Frequency counter | Essential |
| Pulse monitoring | Essential |
| PLL lock monitoring | Very important |
| Jitter measurement | Important |
| Current monitoring | Important |
| Startup testing | Important |

Practical Simplified Interpretation

| Behavior | Likely Radiation Effect |
| --- | --- |
| Small frequency drift | TID |
| One pulse glitch | SET |
| Clock freeze | SEFI |
| Gradual instability | DD/TID |
| Sustained overcurrent | SEL |
| No oscillation | Permanent damage |

Opto-Coupler Radiation-Induced Failures

An opto-coupler contains:

- LED emitter,

- optical path,

- phototransistor/photodiode receiver.

Radiation affects both:

- optical efficiency,

- semiconductor gain.

Optocouplers are especially sensitive to:

displacement damage (DD)

because light-emitting efficiency degrades.

Main Opto-Coupler Failure Types

| Failure Type | What Happens | Radiation Cause | Observable Signature |
| --- | --- | --- | --- |
| CTR Degradation | Current transfer ratio decreases | DD, TID | Output current drops |
| Slower Switching | Propagation delay increases | TID/DD | Delayed output pulse |
| Pulse Distortion | Output waveform changes | SET/TID | Rounded/missing pulses |
| Reduced Optical Efficiency | LED output weakens | DD | Lower output response |
| Increased Leakage | OFF-state leakage rises | TID | Unwanted output current |
| Transient Pulse Error | Temporary wrong switching | SET | False pulse/glitch |
| SEFI-like Stuck State | Output stuck ON/OFF | SEE | No switching response |
| Complete Isolation Failure | Device no longer transfers signal | Severe DD/TID | No output activity |

Important Radiation Effects in Opto-Couplers

| Radiation Effect | Typical Behavior |
| --- | --- |
| DD | LED efficiency loss |
| TID | Gain/leakage degradation |
| SET | Temporary pulse glitch |
| SEFI | Stuck output |
| SEL | Rare but possible in output IC stage |

Most Important Opto-Coupler Radiation Parameter

CTR (Current Transfer Ratio)

Definition

CTR = Output Current / Input LED Current

Radiation usually causes:

CTR ↓

Opto-Coupler Test Methodologies

1. Pulse Transfer Test

Method

Apply square-wave pulses:

0 → 1 → 0

to LED input.

Observe output waveform.

Detects

| Failure | Signature |
| --- | --- |
| SET | Missing/extra pulse |
| TID/DD | Slow edges |
| SEFI | Output stuck |

2. CTR Measurement

Method

Apply fixed LED current.

Measure:

output current

Detects

| Failure | Signature |
| --- | --- |
| DD | CTR reduction |
| TID | Gain degradation |

Example

Before radiation:

CTR = 100%

After:

CTR = 55%

Strong indication of:

- optical degradation.

3. Propagation Delay Test

Method

Measure:

input-to-output delay

Detects

| Failure | Signature |
| --- | --- |
| TID | Delay increase |
| DD | Slower switching |

4. Isolation Integrity Test

Method

Verify:

- proper electrical isolation,

- no unintended conduction.

Detects

| Failure | Signature |
| --- | --- |
| Severe damage | Isolation breakdown |
| Leakage | Increased leakage current |

5. Current Monitoring

Detects

| Observation | Meaning |
| --- | --- |
| Increased current | TID leakage |
| Sustained overcurrent | Possible SEL |

Typical Opto-Coupler Radiation Signatures

| Observation | Likely Effect |
| --- | --- |
| Reduced output amplitude | DD |
| Slower pulse edges | TID/DD |
| Missing pulse | SET |
| Output stuck | SEFI |
| Increased leakage | TID |
| No output | Severe degradation |

Best Detection Strategy for Your System

| Technique | Importance |
| --- | --- |
| Pulse train monitoring | Essential |
| CTR measurement | Essential |
| Propagation delay tracking | Important |
| Output amplitude measurement | Important |
| Current monitoring | Useful |

Operational Amplifier (Op-Amp) Radiation-Induced Failures

Op-amps are highly sensitive because they rely on:

- transistor matching,

- bias currents,

- differential analog stages.

Radiation affects:

- offset,

- gain,

- bandwidth,

- noise,

- bias stability.

Bipolar op-amps are often very sensitive to:

displacement damage.

Main Op-Amp Failure Types

| Failure Type | What Happens | Radiation Cause | Observable Signature |
| --- | --- | --- | --- |
| Offset Voltage Drift | Output shifts from expected value | TID | DC offset error |
| Gain Reduction | Amplification decreases | TID/DD | Lower output amplitude |
| Bandwidth Reduction | Frequency response degrades | TID/DD | Slower response |
| Noise Increase | Output becomes noisy | TID/DD | Increased fluctuations |
| Bias Current Shift | Input current changes | TID | Input instability |
| Slew Rate Reduction | Output changes more slowly | TID | Delayed transitions |
| Transient Glitch | Temporary wrong output | SET | Output spike |
| Saturation/Stuck Output | Output locked high/low | SEFI | No response |
| Oscillation Instability | Amplifier becomes unstable | TID | Ringing/oscillation |
| Increased Current Consumption | Leakage increases | TID | Higher supply current |

Important Radiation Effects in Op-Amps

| Radiation Effect | Typical Op-Amp Behavior |
| --- | --- |
| TID | Offset/gain drift |
| DD | Gain/noise degradation |
| SET | Output transient spike |
| SEFI | Output stuck/frozen |
| SEL | Sustained overcurrent |

Most Important Op-Amp Radiation Problem

Usually:

offset drift

and

noise increase.

Because analog precision is highly sensitive to transistor mismatch.

Op-Amp Test Methodologies

1. DC Offset Test

Method

Apply:

0V input

Measure output.

Detects

| Failure | Signature |
| --- | --- |
| TID | Offset drift |
| Bias degradation | Output shift |

Example

Expected:

0V

Observed:

+120mV

Indicates:

- radiation-induced offset drift.

2. Gain Test

Method

Apply known signal:

VIN

Measure:

VOUT/VIN

Detects

| Failure | Signature |
| --- | --- |
| DD | Gain reduction |
| TID | Transfer degradation |

3. Sine-Wave Response Test

Method

Apply sinusoidal input.

Measure:

- amplitude,

- phase,

- distortion.

Detects

| Failure | Signature |
| --- | --- |
| Bandwidth degradation | Reduced high-frequency response |
| Noise increase | Distorted waveform |

4. Transient Response Test

Method

Apply pulse/square wave.

Observe:

- rise time,

- overshoot,

- settling.

Detects

| Failure | Signature |
| --- | --- |
| SET | Output glitch |
| Slew degradation | Slower response |

5. Noise Measurement

Method

Measure output variance with stable input.

Detects

| Failure | Signature |
| --- | --- |
| DD/TID | Increased noise floor |

6. Current Monitoring

Detects

| Observation | Meaning |
| --- | --- |
| Increased current | TID leakage |
| Sustained overcurrent | SEL |

Typical Op-Amp Radiation Signatures

| Observation | Likely Effect |
| --- | --- |
| Output DC shift | TID |
| Reduced gain | DD/TID |
| One transient spike | SET |
| Output stuck high/low | SEFI |
| Increased noise | DD |
| Higher supply current | TID |
| Oscillation instability | Analog degradation |

Best Detection Strategy for Your System

| Technique | Importance |
| --- | --- |
| DC offset monitoring | Essential |
| Gain verification | Essential |
| ADC feedback measurement | Essential |
| Pulse response testing | Important |
| Noise monitoring | Important |
| Current monitoring | Useful |