ECC = Error Correction Code

ECC is a technique used in memories to:

- detect bit errors,

- correct bit errors automatically.

Very important in:

- NAND Flash,

- DRAM,

- FPGA BRAM,

- SSDs,

- spacecraft memory systems.

Why ECC Is Needed

Radiation can cause:

bit flips

Example:

Stored:

10101010

After SEU:

10100010

ECC adds extra redundant bits so the system can:

- detect corruption,

- identify wrong bit,

- correct it.

ECC Growth Monitoring Meaning

In NAND Flash radiation testing:

Initially:

few or zero corrected errors

After radiation exposure:

more corrected errors appear

This increase is called:

ECC growth

It indicates:

- memory degradation,

- increasing radiation damage,

- worsening retention.

Example

| Radiation Dose | ECC Corrected Errors |
| --- | --- |
| 0 krad | 0 |
| 5 krad | 2 |
| 10 krad | 25 |
| 20 krad | 300 |

Increasing ECC count means:

- cells are becoming unreliable,

- even if data is still recoverable.

Why ECC Monitoring Is Powerful

Because memory can appear:

"working normally"

while internally:

- many bit errors are already occurring.

ECC reveals:

hidden degradation.

Common ECC Types

| ECC Type | Capability |
| --- | --- |
| Parity | Detect only |
| Hamming Code | Correct single-bit |
| BCH | Correct multiple bits |
| Reed-Solomon | Burst error correction |
| LDPC | Advanced NAND/SSD ECC |

Radiation Interpretation

| ECC Behavior | Meaning |
| --- | --- |
| Occasional correction | Normal SEU |
| Gradually increasing corrections | TID/DD degradation |
| Burst corrections | MBU/memory instability |
| ECC unable to correct | Permanent failure/block failure |

In NAND Flash

Modern NAND internally performs:

ECC correction automatically

Radiation testing often monitors:

- number of corrected bits,

- correction rate,

- uncorrectable blocks.

These are key health indicators.

TID and DD are cumulative effects

meaning:

damage accumulates slowly with radiation exposure time/dose

Unlike:

- SEU,

- SET,

- SEL,

which happen suddenly from one particle strike.

Difference Between SEE vs TID/DD

| Type | Nature | Time Scale |
| --- | --- | --- |
| SEU/SET/SEL | Instantaneous | ns to ms |
| TID | Gradual accumulation | hours/months/years |
| DD | Gradual crystal damage accumulation | long-term |

Then How Are TID/DD Tested?

You do:

parameter trending over time/dose

Instead of:

"did one event happen?"

you ask:

"how is device behavior slowly changing?"

General TID/DD Testing Philosophy

Step 1 — Measure Initial Baseline

Before irradiation:
measure:

- current,

- voltage,

- timing,

- frequency,

- gain,

- BER,

- CTR,

- threshold voltage,

- memory retention,
etc.

This becomes:

reference data

Step 2 — Irradiate Incrementally

Example:

2 krad

5 krad

10 krad

20 krad

or neutron fluence for DD.

After each step:

- remeasure parameters.

Step 3 — Trend the Degradation

You look for:

- drift,

- increased noise,

- increased current,

- slower timing,

- retention loss.

TID Example — Op-Amp

Initial:

offset = 2 mV

After 20 krad:

offset = 45 mV

This gradual change indicates:

TID degradation

DD Example — Optocoupler

Initial:

CTR = 100%

After neutron exposure:

CTR = 55%

That gradual efficiency loss indicates:

displacement damage

How Each Device Is Checked for TID/DD

1. FPGA

TID/DD Indicators

| Parameter | Meaning |
| --- | --- |
| Supply current increase | Leakage/TID |
| Timing margin reduction | TID |
| BRAM error increase | TID/DD |
| PLL instability | TID |
| Configuration failure | Severe degradation |

Methodology

Run:

- counters,

- BRAM tests,

- timing tests,

- current monitoring

continuously while dose accumulates.

2. NAND Flash / EEPROM

TID/DD Indicators

| Parameter | Meaning |
| --- | --- |
| Retention loss | Oxide degradation |
| ECC correction increase | Weakening cells |
| Bad block growth | Permanent damage |
| Write/erase failure | Tunnel oxide wear |

Methodology

Repeated:

write → wait → read → compare

after increasing dose.

3. ADC/DAC

TID/DD Indicators

| Parameter | Meaning |
| --- | --- |
| Offset drift | TID |
| Gain drift | TID/DD |
| Noise increase | DD |
| INL/DNL degradation | Analog degradation |

Methodology

Continuously apply:

- known voltage/reference.

Track:

measured value vs expected value

over time.

4. Op-Amp

TID/DD Indicators

| Parameter | Meaning |
| --- | --- |
| Offset increase | TID |
| Gain reduction | DD |
| Noise increase | DD/TID |
| Slew reduction | TID |

Methodology

Periodic:

- DC offset measurement,

- gain measurement,

- pulse response test.

5. MOSFET

TID/DD Indicators

| Parameter | Meaning |
| --- | --- |
| Threshold shift | TID |
| Leakage current increase | TID |
| RDS(on) increase | DD |
| Switching slowdown | DD/TID |

Methodology

Repeatedly measure:

VTH

IDSS

RDS(on)

during accumulated dose.

6. Optocoupler

TID/DD Indicators

| Parameter | Meaning |
| --- | --- |
| CTR decrease | DD |
| Delay increase | TID |
| Output amplitude reduction | DD |

Methodology

Apply fixed LED current:
measure output current over time.

7. XTAL Oscillator

TID/DD Indicators

| Parameter | Meaning |
| --- | --- |
| Frequency drift | TID |
| Increased jitter | DD/TID |
| Startup failure | Severe degradation |

Methodology

Continuously count:

clock frequency

during irradiation.

Core Concept

SEE detection:

Did an event happen?

TID/DD detection:

How much did parameters drift over accumulated exposure?

What Makes TID/DD Difficult

Because:

- no sudden obvious failure,

- degradation may be slow,

- devices may still "work" while becoming unreliable.

Example:

ECC corrections slowly rising

means damage already accumulating.

Best Practical Method

For your system:

continuous telemetry logging

Store over time:

- current,

- voltage,

- frequency,

- error count,

- retention failures,

- CTR,

- ADC offset,

- FPGA mismatch count.

Then compare:

parameter vs accumulated dose

That is the standard way TID/DD are characterized.

krad is Radiation Dose, not Time

rad means:

Radiation Absorbed Dose

1 rad = 0.01 joule/kg

and:

1 krad = 1000 rad

Important

The time required to reach:

1 krad

10 krad

100 krad

depends on:

dose rate

Formula

Time = Total Dose / Dose Rate

Example 1

If irradiation facility provides:

1 krad/hour

Then:

| Total Dose | Time Required |
| --- | --- |
| 1 krad | 1 hour |
| 10 krad | 10 hours |
| 50 krad | 50 hours |

Example 2

If dose rate is:

10 rad/sec

Then:

1 krad = 100 seconds

because:

1000 rad / 10 rad/s = 100 s

Space vs Laboratory

Huge difference.

In Real Space

Dose accumulates slowly.

Typical:

- months,

- years.

Example:

10–50 krad over mission lifetime

In Radiation Facility

They accelerate exposure.

Example:

50 krad in few hours

using:

- gamma source,

- proton beam,

- heavy ion beam.

Why Accelerated Testing Is Done

Because waiting:

10 years

for natural accumulation is impractical.

So labs use:

accelerated dose-rate testing.

Typical Radiation Sources and Dose Rates

| Source | Typical Dose Rate |
| --- | --- |
| Co-60 Gamma | rad/hr → krad/hr |
| X-ray source | variable |
| Proton beam | depends on flux |
| Heavy ion beam | SEE-focused |
| Space environment | very slow |

Important Difference

| Effect | Depends on |
| --- | --- |
| TID | Total accumulated dose |
| DD | Particle fluence over time |
| SEE | Single particle strike |

Example TID Test Sequence

Typical lab procedure:

| Step | Dose |
| --- | --- |
| Initial | 0 krad |
| Step 1 | 2 krad |
| Step 2 | 5 krad |
| Step 3 | 10 krad |
| Step 4 | 20 krad |

After each step:

- pause irradiation,

- measure device parameters,

- continue exposure.

Important Practical Point

Sometimes:

dose rate itself affects behavior

High dose-rate testing may produce:

- different annealing behavior,

- different trapped charge dynamics.

So:

100 krad in 1 hour

is not always identical to:

100 krad over 5 years

This is called:

dose-rate effect

and is important in radiation qualification.