Got it 👍 — you want “what each signal is + how it works” in a simple, practical way (not just definitions).
Let’s do that clearly for the main signals of the ADS8588H.

🔷 1. 🟢 CONVSTA / CONVSTB

👉 What it is:

A start button for conversion

👉 How it works:

- You give a LOW pulse

- ADC starts converting analog → digital

🧠 Key point:

- CONVSTA → AIN1–AIN4

- CONVSTB → AIN5–AIN8

👉 Pulse both → all 8 channels convert

🔷 2. 🔴 BUSY

👉 What it is:

A status signal

👉 How it works:

- Goes HIGH → ADC is converting

- Goes LOW → conversion finished

🧠 Key point:

👉 Only read data when BUSY = LOW

🔷 3. 🔵 CS (Chip Select)

👉 What it is:

An enable switch

👉 How it works:

- LOW → ADC is active

- HIGH → ADC disconnects from bus (no output)

🧠 Key point:

👉 Without CS LOW, ADC won’t give data

🔷 4. 🟡 RD (Read)

👉 What it is:

A data request signal

👉 How it works:

- Each time RD goes LOW
→ ADC outputs next channel data

🧠 Key point:

👉 Multiple RD pulses = AIN1 → AIN2 → AIN3 …

🔷 5. 🟣 DB[15:0]

👉 What it is:

A 16-bit data output bus

👉 How it works:

- ADC places digital value here

- You read it using MCU/FPGA

🔷 6. 🟠 FRSTDATA

👉 What it is:

A marker signal

👉 How it works:

- Goes HIGH when output = AIN1

🧠 Key point:

👉 Helps you know where sequence starts

🔷 7. ⚪ SCLK (Serial Clock)

👉 What it is:

A clock for serial mode

👉 How it works:

- Each clock pulse → 1 bit comes out

🔷 🔥 FULL WORKING (Putting everything together)

1. CONVSTA/B pulse → start conversion

2. BUSY = HIGH → ADC working

3. BUSY = LOW → data ready

4. CS = LOW → enable ADC

5. RD pulse → AIN1

6. RD pulse → AIN2

7. RD pulse → AIN3 …

🧠 SUPER SIMPLE ANALOGY

| Signal | Real-life meaning |
| --- | --- |
| CONVST | Start exam |
| BUSY | Students writing |
| CS | Call this student |
| RD | “Next answer please” |
| DB | Answer sheet |
| FRSTDATA | First student indicator |

🎯 FINAL UNDERSTANDING

👉 Each signal has a job in sequence:

- CONVST → Start

- BUSY → Wait

- CS → Enable

- RD → Read step-by-step

✅ One-line summary

👉
These signals work together to start conversion, indicate completion, enable the ADC, and sequentially output channel data.

If you want next level clarity, I can:
✅ Draw one clean timing diagram with all signals labeled
✅ Or explain what happens inside ADC during BUSY (SAR process)