A user normally does not inspect ECC bytes manually. The controller or ECC engine does that automatically.

However, if you want to check for bit flips yourself, the process is:

1. Read Data Area

Example:

Data read:

55 AA 55 AA

2. Read OOB (Spare Area)

The page also contains ECC bytes in the OOB area.

Example:

OOB:

3A 7C 91 ...

3. Recalculate ECC

Run the same ECC algorithm (Hamming, BCH, LDPC, etc.) on the read data:

55 AA 55 AA

↓

Calculated ECC = 3A 7C 91

4. Compare

Stored ECC      = 3A 7C 91

Calculated ECC = 3A 7C 91

Match:

No bit errors

If:

Stored ECC      = 3A 7C 91

Calculated ECC = 3B 7C 91

then:

Bit error detected

How does the user know where ECC bytes are?

The NAND controller defines the layout.

Typical example for a 2 KB page:

Main area: 2048 bytes

OOB area: 64 bytes

--------------------------------

Byte 0-1   : Bad Block Marker

Byte 2-15  : Metadata

Byte 16-63 : ECC bytes

The exact positions depend on the controller and ECC scheme.

In Linux NAND systems

The kernel exposes ECC statistics:

cat /sys/class/mtd/mtd0/ecc_stats

or via:

dmesg

You may see:

Corrected: 12

Failed: 0

which means ECC has already detected and corrected 12 bit flips.

In simulation

A common verification approach is:

- Program data.

- Inject a bit flip:

- 55 -> 51

- Read page.

- Run ECC checker.

- Verify:

- Correctable error = 1

- Corrected data = 55

So the user identifies bit flips by either:

- Comparing stored ECC bytes with recalculated ECC, or

- Letting the ECC engine report the number of corrected/uncorrectable errors. The second approach is what real systems almost always use.

***************************************************************************

There are two different situations:

1. Your BlueDBM NAND model

From everything you've shown, the NAND model itself does not appear to implement ECC correction. It simply stores and returns bytes.

So there is probably nothing to "turn off".

You can check by searching the source for:

ECC

ecc

BCH

LDPC

HAMMING

If no ECC module exists, then ECC is already effectively off.

2. If you want to verify bit errors

The simplest method is:

Write

write_data(8'h55);

Corrupt memory manually

Inside the NAND model, change stored data:

memory[...] = 8'h51;

Read

read_byte(read_val);

if (read_val != 8'h55)

$display("BIT ERROR DETECTED");

Output:

Expected = 55

Read     = 51

BIT ERROR DETECTED

Count bit flips

Suppose:

Expected = 55 = 01010101

Read     = 51 = 01010001

Compute:

reg [7:0] diff;

integer bit_errors;

diff = 8'h55 ^ read_val;

Result:

01010101

XOR

01010001

=

00000100

One bit differs.

You can count them:

bit_errors = 0;

for (i=0; i<8; i=i+1)

if(diff[i])

bit_errors = bit_errors + 1;

$display("Bit Errors = %0d", bit_errors);

If you want ECC-like behavior

Store the expected data in a reference array:

expected_mem[addr] = 8'h55;

Then after read:

if(read_val !== expected_mem[addr])

begin

$display("ERROR at addr=%0d Expected=%02h Read=%02h",

addr,

expected_mem[addr],

read_val);

end

This is the usual first step before implementing a real ECC engine.

For the BlueDBM model specifically, tell me where the memory array is declared in nand_die_model.v (the reg [...] mem ... line), and I can show the exact line to modify so a programmed 55 comes back as 51 during read.

*************************************************************************

That ECC directory in BlueDBM is likely the ECC engine implementation, not part of the NAND model itself.

From the repository structure and filenames, it appears to implement BCH ECC (Bose–Chaudhuri–Hocquenghem), which is commonly used with NAND flash to detect and correct multiple bit errors. BCH ECC stores parity bytes alongside the data and can correct several flipped bits per ECC block. (GitLab)

The typical flow is:

Write Data

↓

ECC Encode

↓

Data + ECC stored

Read Data + ECC

↓

ECC Decode

↓

Correct errors if possible

The ECC engine calculates ECC bytes during writes and, during reads, compares the stored ECC with a newly calculated ECC to detect/correct errors. (codebrowser.dev)

How to "turn off" ECC

You need to determine whether your testbench is using the ECC module at all.

Search the BlueDBM project for:

ECC

bch

encode

decode

correct

or module names instantiated from the ECC directory.

If the data path is:

Testbench

↓

nand_model

then ECC is probably not involved.

If the path is:

Testbench

↓

Controller

↓

ECC

↓

nand_model

then ECC may be active.

How to check whether ECC is correcting errors

A good experiment:

- Program 55.

- Manually change NAND memory to 51.

- Read back.

Results:

Read = 51

→ ECC is not correcting.

Read = 55

→ ECC corrected the bit flip.

ECC error reported

→ ECC detected the error.

Where to inject errors

Since you're using nand_die_model.v, the best place is inside the stored page data.

Example:

55 = 01010101

51 = 01010001

One bit flipped.

Then:

read_byte(read_val);

if(read_val != 8'h55)

$display("Bit error detected");

Next step

Open nand_die_model.v and find where this message is printed:

Latch Data (0 : 0 : 0 + 0) = 55

The memory array being written at that point is the place where you can inject bit errors. If you paste that code section, I can show exactly where to force:

55 → 51

so the error appears during the read operation.

************************************************************

To use the BlueDBM ECC, first you need to identify the top-level ECC module.

Go to the ECC directory and find lines like:

module bch_encoder(...)

module bch_decoder(...)

module ecc_top(...)

Without knowing the module names and ports, nobody can wire it correctly.

Step 1

Open the ECC folder and show:

controller/src/hw_artix/ECC/

file list.

For example:

bch_encoder.v

bch_decoder.v

ecc_encoder.v

ecc_decoder.v

...

Step 2

Open the likely top-level file and paste:

module xxxx(

...

);

including all port declarations.

Step 3

Then the connection will look roughly like:

wire [DATA_W-1:0] data_in;

wire [ECC_W-1:0]  ecc_out;

ecc_encoder enc (

.data_in(data_in),

.ecc_out(ecc_out)

);

and for reads:

wire [DATA_W-1:0] corrected_data;

wire error_detected;

ecc_decoder dec (

.data_in(read_data),

.ecc_in(stored_ecc),

.corrected_data(corrected_data),

.error(error_detected)

);

Step 4: Error Injection Test

Write:

55 AA 55 AA

Generate ECC.

Store both data and ECC.

Before reading, intentionally flip one bit:

read_data[2] = ~read_data[2];

Example:

55 -> 51

Then feed:

corrupted data + original ECC

to the decoder.

Expected result:

Input          = 51

ECC            = original

Corrected Data = 55

Error Count    = 1

At the moment, I need the actual ECC module names and ports from the BlueDBM ECC directory. Paste the file list or the first 50 lines of the main ECC file, and I can show the exact instantiation code.

*******************************************************************

You cannot simply "add ECC" to this testbench by inserting a few lines.

Your testbench currently does:

Write data → NAND

Read data ← NAND

ECC requires:

Write data

↓

ECC Encoder

↓

Data + ECC

Read data + ECC

↓

ECC Decoder

↓

Corrected data

For now, simulate ECC behavior

Add an expected memory:

At the top:

reg [7:0] expected_mem [0:2047];

integer bit_errors;

reg [7:0] diff;

During write

Replace:

if (i[0])

write_data(8'hAA);

else

write_data(8'h55);

with:

if (i[0]) begin

expected_mem[i] = 8'hAA;

write_data(8'hAA);

end

else begin

expected_mem[i] = 8'h55;

write_data(8'h55);

end

During read

Instead of:

read_byte(read_val);

use:

read_byte(read_val);

// Inject artificial bit flip

if(i==2)

read_val = read_val ^ 8'b00000100;

if(read_val != expected_mem[i]) begin

diff = read_val ^ expected_mem[i];

bit_errors = 0;

if(diff[0]) bit_errors = bit_errors + 1;

if(diff[1]) bit_errors = bit_errors + 1;

if(diff[2]) bit_errors = bit_errors + 1;

if(diff[3]) bit_errors = bit_errors + 1;

if(diff[4]) bit_errors = bit_errors + 1;

if(diff[5]) bit_errors = bit_errors + 1;

if(diff[6]) bit_errors = bit_errors + 1;

if(diff[7]) bit_errors = bit_errors + 1;

$display("ECC: Error detected");

$display("Expected=%02h Read=%02h BitErrors=%0d",

expected_mem[i], read_val, bit_errors);

// Simulate ECC correction

read_val = expected_mem[i];

$display("ECC corrected to %02h", read_val);

end

Example output

Expected=55

Read=51

BitErrors=1

ECC corrected to 55

For the actual BlueDBM BCH ECC, I need the contents of:

controller/src/hw_artix/ECC

especially the module names such as:

module BCHEncoder(...)

module BCHDecoder(...)

or similar.

Paste the file list from the ECC directory and I can show exactly how to instantiate the real encoder/decoder in your testbench. Right now we don't know the ECC module interface.

------------------------------------------------------------------------------------------------------------

For a real NAND such as MT29F128G08 (Micron 128-Gb NAND), the NAND chip itself typically does not contain a user-visible ECC engine that computes and corrects data for you. Instead:

NAND Flash  <---->  NAND Controller

|

+--> ECC Engine

The ECC algorithm is usually implemented in the controller, FPGA, SoC, SSD controller, or NAND driver.

Older NAND

Older SLC NAND often used:

Hamming Code

Typical capability:

1-bit correction per 512-byte sector

Modern MLC/TLC NAND

Devices like MT29F128G08 generally require much stronger ECC:

BCH ECC

or in newer SSDs:

LDPC ECC

Typical requirement:

4-bit correction / 512 bytes

8-bit correction / 512 bytes

24-bit correction / 1 KB

40-bit correction / 1 KB

depending on the NAND generation and operating conditions.

Is there a single equation?

Not really.

Hamming

Parity bits are computed from XOR combinations:

p1 = d1 ⊕ d2 ⊕ d4 ⊕ d5 ...

p2 = d1 ⊕ d3 ⊕ d4 ⊕ d6 ...

BCH

Uses polynomial arithmetic over a finite field:

GF(2^m)

Generator polynomial:

g(x)

Encoding:

codeword = data(x) · x^(n-k) mod g(x)

The remainder becomes the ECC bytes.

LDPC

Uses a sparse parity-check matrix:

H · c^T = 0

and iterative decoding algorithms.

What does Micron expect?

For the MT29F128G08 family, the datasheet normally specifies an ECC requirement such as:

Host must provide ECC capable of correcting

at least N bits per 512-byte sector.

The NAND stores:

Data Area

+

ECC bytes in OOB (spare area)

but does not care whether those ECC bytes came from BCH, Hamming, or another compliant ECC engine.

If you are building an FPGA controller

The usual choice is:

BCH ECC

because:

- Widely used for raw NAND

- Hardware-friendly

- Corrects multiple bit errors

- Supported by many open-source implementations

The BlueDBM ECC directory you found is very likely implementing BCH ECC for this reason.

-----------------------------------------------------------------------------------------------

For testing your NAND model, I would not start with BCH or LDPC.

Start simple.

### Option 1: No ECC (best first step)

Write pattern:

55 AA 55 AA

Read back:

if(read_val != expected_val)

$display("ERROR");

Inject errors manually:

read_val = read_val ^ 8'b00000100;

Verify that your testbench detects:

Expected = 55

Read = 51

This proves your NAND read/write path works.

### Option 2: Simple Hamming ECC (best for learning)

Advantages:

- Easy Verilog implementation

- Small logic

- Can correct 1-bit errors

- Can detect 2-bit errors

Example:

55

↓

Bit flip

↓

51

↓

Hamming corrects

↓

55

Good for FPGA verification and understanding ECC.

### Option 3: BCH ECC (best for realistic NAND)

This is what most NAND controllers use.

Capabilities:

Correct 4 bits

Correct 8 bits

Correct 24 bits

...

BlueDBM's ECC folder is very likely BCH-based.

Use BCH if:

You want realistic NAND controller testing

### Option 4: LDPC

Used in:

- Modern SSDs

- High-density TLC/QLC NAND

But:

Very complex

Not suitable as first ECC test

### Recommendation

For your current stage:

1. Get NAND read/program working

2. Inject bit flips manually

3. Add Hamming ECC

4. Move to BlueDBM BCH ECC

A practical test sequence would be:

Write:    55

Stored:   55

Inject:   55 -> 51

Read:     51

ECC:      detects 1-bit error

ECC:      corrects to 55

Compare:  PASS

That gives you a complete ECC verification flow without immediately dealing with the complexity of BCH decoding. Once that works, integrating the BCH modules from the BlueDBM ECC directory becomes much easier.